# UNIT 1

PLANNING FOR WATER SUPPLY SYSTEM

IMPORTANCE AND NECESSITY FOR PLANNED WATER SUPPLIES

Next to the air, the other important requirement for human life to exists is water.Water is vailable in various forms such as rivers, lake, streams etc. The earliest civilizations organized on the banks of major river systems and required water for drinking, bathing, cooking etc. But with the advancement of civilization the utility of water enormously increased and now such a stage has come that without well organized public water supply scheme, it is impossible to run the present civic life and the develop the towns. The importance of water from only a quantity viewpoint was recognized from the earliest days and the importance of quality come to be recognized gradually in the later days. The earliest recorded knowledge of water quality and its treatment are found in Sanskrit literature “Sushuri Sanhita” compiled about 2000 B.C. It deals with storage of drinking water in copper vessels, exposure to sunlight, filtering through charcoal, sand etc.

The correlation between water quality and incidence of diseases was first established in 1849 by Dr. John snow when cholera appeared in London during the summer and 14,600 deaths were reported. But Dr. snow unable to convince the authorities and public with the evidence of available data. The water borne diseases like typhoid, dysentery, cholera etc the concept of water borne diseases was well accepted by 1900. Another striking example was reported from Uttarpradesh by W.H.O (World Health Organisation) in 1963, there the death rate by chorera decreased by 74.1%, Thyphoid fever by 63.6% , by dysentery 23.1% and diarria by 63.6%. All these were achieved by drinking water treatment.

# NEED FOR PROTECTED WATER SUPPLY

Protected water supply means the supply of water that is treated to remove the impurities and made safe to public health. Water may be polluted by physical and bacterial agents. Water is also good carrier of disease causing germs. The causes of outbreak of epidemics are traced to pollute water and poor sanitation hospital are continued to be flooded with the sick due to ignorance about health continues to be profound. However during the last few decades, improvements in the public health protection by supplying safe water and sanitation to all the people in the developing countries. In 1977, united nations declare to launch a movement known as “HEALTH FOR ALL BY THE YEAR 2000 A.D.” India is also a signatory to that conference. The working group appointed by the planning commission while suggesting strategies for achieving the above goal emphasized that potable water from protected water supply should be made available to the entire population. Pure and whole some water is to be supplied to the community alone can bring down the morbidity rates At present, only 16 percent of towns in our country are equipped with water supply works serving about five percent of population of the whole of country. India has get to make serious efforts to make the treated water available to the most of its population so as to minimise the water borne diseases. Therefore protected water supply is a SIN QUO NON of public health of a community.

The objectives of the community water supply system are

- to provide whole some water to the consumers for drinking purpose.

- to supply adequate quantity to meet at least the minimum needs of the individuals

- to make adequate provisions for emergencies like fire fighting, festivals, meeting etc

- to make provision for future demands due to increase in population, increase in standard of living, storage and conveyance

- to prevent pollution of water at source, storage and conveyance

- to maintain the treatment units and distribution system in good condition with adequate staff and materials

- to design and maintain the system that is economical and reliable

# WHOLE SOME WATER

Absolutely pure water is never found in nature and which contains only two parts of hydrogen and one part of oxygen by volume. But the water found in nature contains number of impurities in varying amounts. The rainwater which is originally pure, also absorbs various gases, dust and other impurities while filling. This water when moves on the ground further carries silt, organic and inorganic impurities. The removal of the turbidity, odour and smell is considered as good and removal of dissolved substances is considered as “chemically pure”. But removal of substances like calcium, magnesium Iron, Zinc etc completely is not good for health. These minerals are required for tissue growth and some act as propylatic in preventing diseases. Therefore wholesome water is defined as the water which containing the minerals in small quantities at requisite levels and free from harmful impurities Chemically pure water is also corrosive but not whole some water. The water that is fit for drinking safe and agreeable is called potable water.

The following are the requirements of wholesome water.

- It should be free from bacteria

- It should be colourless and sparkling

- It should be tasty, odour free and cool

- It should be free from objectionable matter

- It should not carrode pipes

- It should have dissolved oxygen and free from carbonic acid so that it may remain fresh

# STATUS OF PROTECTED WATER SUPPLY IN INDIA

Lack of safe drinking water in India is still a problem in many areas of the country. As per the U.N. report (1983), town and cities only 86% of the urban population have some provision for protected water supplies. Only one village out of ten has safe drinking water. It is important to note that 80% of India’s population live in villages and only 6 crores have access for safe water.

# PLANNING AND EXECUTION OF MODERN WATER SUPPLY SCHEMES

After british rule in our country, investments made in successive five year plans for planned development towards urban and Rural water supply and sanitation. Because of shortage of funds and some other reasons were responsible for slow growth of water supply facilities during the last five year plans. There are many central, state and International agencies coordinating and executing the urban and rural water supply schemes in the country

- Central public health and environment organization under the ministry of works and housing formulates schemes and provide assistance to states planning and development.

- National environment engineering research institute (NEERI) is a research institute of Govt. of India, conducts water quality surveys and suggests treatment processes and also provides design of treatment and distribution system

- CSIR laboraties (council of scientific and industrial research) provide testing facilities for water quality maintenance.

- Central ground water bound, Geological survey of India, national geographical research institute (NGRI) arte engaged afflicted by fluoride Iron, Manganese etc.

- Technology missions were launched by Govt. of India in 1986 with submissions on control of flows

- Bharat Heavy Electricals Limited is providing technology in such special processes like Reverse Electro-Dialysis.

- Public Health Engineering Departments undertake execution of large schemes for water supply and sanitation.

- State ground water department evaluates the quality and quantity of ground water all over the state.

- Panchayat Raj Engineering department of state Govt. is the model agency for providing water supply and sanitation facilities in rural and urban panchayats.

- A.P. State council of science and technology is engaged in assessing the status and quality of drinking water availability and requirements in selected areas.

- Medium and major irrigation departments of Govt. undertake multipurpose schemes in the state with component of water supply along with Hydroelectric, irrigation, navigation, tourism and other services.

- Educational institutions – many engineering colleges offer course in environmental engineering water supply and sanitary engineering at degree and postgraduate levels. Polytechnics and vocational courses conduct courses in water supply engineering to train the technicians and engineers to the growing demand

- International organizations like UNICEF (United Nations Health Organization) provide technical assistance and knowledge on water supply schemes working in specific problem areas.

- There are many Non-Governmental organizations (N.G.O) like water development society, environmental protection societies operating in limited areas with donations and contributions by public and Govt.

# SUMMARY:

- Nearly 80% of the communicable diseases are transmitted through drinking water. Hence to protect the health of the community, protected water supply should be made available for all

- The water contains minerals like calcium, magnesium, Iron, Zinc etc in small quantities at requisite levels and free from harmful impurities is called whole some water. It promote better health and is not injurious in anyway.

- To achieve the goal of “Health for all by 2000 A.D.” several states, National, international and non-governmental agencies are working to make wholesome water available to all.

# WATER DEMANDS

VARIOUS TYPES OF WATER DEMANDS

While designing the water supply scheme for a town or city, it is necessary to determine the total quantity of a water required for various purposes by the city. As a matter of fact the first duty of the engineer is to determine the water demand of the town and then to find suitable water sources from where the demand can be met. But as there are so many factors involved in demand of water, it is not possible to accurately determine the actual demand. Certain empirical formulae and thumb rules are employed in determining the water demand, which is very near to the actual demand.

Following are the various types of water demands of a city or town:

- Domestic water demand

- Industrial demand

- Institution and commercial demand

- Demand for public use

- Fire demand

- Loses and wastes

# DOMESTIC WATER DEMAND

The quantity of water required in the houses for drinking, bathing, cooking, washing etc is called domestic water demand and mainly depends upon the habits, social status, climatic conditions and customs of the people. As per IS: 1172-1963, under normal conditions, the domestic consumption of water in India is about 135 litres/day/capita. But in developed countries this figure may be 350 litres/day/capita because of use of air coolers, air conditioners, maintenance of lawns, automatic household appliances.

The details of the domestic consumption are

- Drinking	5 litres

- Cooking	5 litres

- Bathing	55 litres

- Clothes washing	20 litres

- Utensils washing	10 litres

- House washing	10 litres

135 litres/day/capita

# INDUSTRIAL DEMAND

The water required in the industries mainly depends on the type of industries, which are existing in the city. The water required by factories, paper mills, Cloth mills, Cotton mills, Breweries, Sugar refineries etc. comes under industrial use. The quantity of water demand for industrial purpose is around 20 to 25% of the total demand of the city.

# INSTITUTION AND COMMERCIAL DEMAND

Universities, Institution, commercial buildings and commercial centers includingoffice buildings, warehouses, stores, hotels, shopping centers, health centers, schools,temple, cinema houses, railway and bus stations etc comes under this category. As per IS:1172- 1963, water supply requirements for the public buildings other than residences as follows.

# DEMAND FOR PUBLIC USE

Quantity of water required for public utility purposes such as for washing and sprinkling on roads, cleaning of sewers, watering of public parks, gardens, public fountains etc comes under public demand. To meet the water demand for public use, provision of 5% of the total consumption is made designing the water works for a city.

# FIRE DEMAND

Fire may take place due to faulty electric wires by short circuiting, fire catching materials, explosions, bad intension of criminal people or any other unforeseen mishappenings. If fires are not properly controlled and extinguished in minimum possible time, they lead to serious damage and may burn cities. All the big cities have full fire- fighting squads. As during the fire breakdown large quantity of water is required for throwing it over the fire to extinguish it, therefore provision is made in the water work to supply sufficient quantity of water or keep as reserve in the water mains for this purpose. In the cities fire hydrants are provided on the water mains at 100 to 150 m apart for fire demand. The quantity of water required for fire fighting is generally calculated by using different empirical formulae. For Indian conditions kuichings formula gives satisfactory results.

Q=3182 √p

Where ‘Q’ is quantity of water required in litres/min ‘P’ is population of town or city in thousands

# LOSES AND WASTES

All the water, which goes in the distribution, pipes does not reach the consumers. The following are the reasons

- Losses due to defective pipe joints, cracked and broken pipes, faulty valves and fittings.

- Losses due to, consumers keep open their taps of public taps even when they are not using the water and allow the continuous wastage of water

- Losses due to unauthorised and illegal connections

While estimating the total quantity of water of a town; allowance of 15% of total quantity of water is made to compensate for losses, thefts and wastage of water

# PER CAPITA DEMAND

If ‘Q’ is the total quantity of water required by various purposes by a town per year and ‘p’ is population of town, then per capita demand will be

Q

Per capita demand =	litres/day

P x 365

Per capita demand of the town depends on various factors like standard of living, no. and type of commercial places in a town etc. For an average Indian town, the requirement of water in various uses is as under

  - Domestic purpose	135 litres/c/d

  - Industrial use	40 litres/c/d

  - Public use	25 litres/c/d

  - Fire Demand	15 litres/c/d

  - Losses, Wastage

and thefts	55 litres/c/d

Total : 270 litres/capita/day

The total quantity of water required by the town per day shall be 270 multiplied with the total population in litres/day.

# FACTORS AFFECTING PER CAPITA DEMAND

The following are the main factors affecting for capita demand of the city or town.

- Climatic conditions : The quantity of water required in hotter and dry places is

more than cold countries because of the use of air coolers, air conditioners, sprinkling of water in lawns, gardens, courtyards, washing of rooms, more washing of clothes and bathing etc. But in very cold countries sometimes the quantity of water required may be more due to wastage, because at such places the people often keep their taps open and water continuously flows for fear of freezing of water in the taps and use of hot water for keeping the rooms warm.

- Size of community : Water demand is more with increase of size for town because more water is required in street washing, running of sewers, maintenance of parks and gardens.

- Living standard of the people : The per capita demand of the town increases with

the standard of living of the people because of the use of air conditioners, room coolers, maintenance of lawns, use of flush, latrines and automatic home appliancesetc.

- Industrial and commercial activities : As the quantity of water required in certain industries is much more than domestic demand, their presence in the town will enormously increase per capita demand of the town. As a matter of the fact the water required by the industries has no direct link with the population of the town.

- Pressure in the distribution system: The rate of water consumption increase in the pressure of the building and even with the required pressure at the farthest point, the consumption of water will automatically increase. This increase in the quantity is firstly due to use of water freely by the people as compared when they get it scarcely and more water loss due to leakage, wastage and thefts etc.

- System of sanitation: Per capita demand of the towns having water carriage system will be more than the town where this system is not being used.

- Cost of water: The cost of water directly affects its demand. If the cost of water is more, less quantity of water will be used by the people as compared when the cost is low.

# VARIATIONS IN DEMAND

The per capita demand of town is the average consumption of water for a year. In practice it has been seen that this demand doesnot remain uniform throughout the year but it various from season to season, even hour to hour.

# SEASONAL VARIATIONS

The water demand varies from season to season. In summer the water demand is maximum, because the people will use more water in bathing, cooling, lawn watering and

street sprinkling. This demand will becomes minimum in winter because less water will be used in bathing and there will be no lawn watering. The variations may be upto 15% of the average demand of the year.

# DAILY VARIATIONS

This variation depends on the general habits of people, climatic conditions and character of city as industrial, commercial or residential. More water demand will be on

Sundays and holidays due to more comfortable bathing, washing etc as compared to other working days. The maximum daily consumption is usually taken as 180% of the average consumption.

# HOURLY VARIATIONS

On Sundays and other holidays the peak hours may be about 8 A.M. due to late awakening where as it may be 6 A.M. to 10 A.M. and 4 P.M. to 8 P.M. and minimum flow may be between 12P.M. to 4P.M. when most of the people are sleeping. But in highly industrial city where both day and night shifts are working, the consumption in night may be more. The maximum consumption may be rise upto 200% that of average daily demand.

The determination of this hourly variations is most necessary, because on its basis the rate of pumping will be adjusted to meet up the demand in all hours.

# DESIGN PERIOD

The complete water supply project includes huge and costly constructions such as

dams, reservoirs, treatment works and network of distribution pipelines. These all works cannot be replaced easily or capacities increased conveniently for future expansions.

While designing and constructing these works, they should have sufficient

capacity to meet future demand of the town for number of years. The number of years for which the designs of the water works have been done is known as design period. Mostly water works are designed for design period of 22-30 years, which is fairly good period.

# TOTAL REQUIREMENT OF WATER FOR A TOWN OR A CITY

Total quantity of water required by a town or a city per day shall be 270 multiplied with the total population in litres/day.

# POPULATION FORECASTING METHODS AND PROBLEMS

When the design period is fixed the next step is to determine the population of a town or city population of a town depends upon the factors like births, deaths, migration and annexation. The future development of the town mostly depends upon trade expansion, development industries, and surrounding country, discoveries of mines,construction of railway stations etc may produce sharp rises, slow growth, stationary conditions or even decrease the population. For the prediction of population, it is better to study the development of other similar towns, which have developed under the same circumstances, because the development of the predicted town will be more or less on the same lines.

The following are the standard methods by which the forecasting population is done.

- Arithmetical increase method

- Geometrical increase method

- Incremental increase method

- Simple graph method

- Decrease rate of growth method

- Comparative graph method

- The master plan method

Problem: The following data have been noted from the census department.

# YEAR POPULATION

1940 8000

1950 12000

1960 17000

1970 22500

Find the probable population in the year 1980, 1990 and 2000.

# ARITHEMATICAL INCREASE METHOD

This method is based on the assumption that the population is increasing at a

constant rate. The rate of change of population with time is constant. The population after

‘n’ decades can be determined by the formula.

Pn = P + n.c where

P → population at present n → No. of decades

c → Constant determined by the average

of increase of ‘n’ decades

# YEAR POPULATION INCREASE IN POPULATION

1940 8000 ---

1950 12000 4000

1960 17000 5000

1970 22500 5500

TOTAL 14500

AVERAGE 4833

# Solution:

YEAR POPULATION

1980 22500 + 1 x 4833 = 27333

1990 22500 + 2 x 4833 = 32166

2000 22500 + 3 x 4833 = 36999

# GEOMETRICAL INCREASE METHOD

This method is based on the assumption that the percentage increase in population

from decade to decade remains constant. In this method the average percentage of growth of last few decades is determined, the population forecasting is done on the basis that percentage increase per decade will be the same.

The population at the end of ‘n’ decades is calculated by

Pn = P

where

P → population at present

C → average percentage of growth

of ‘n’ decades

# Year Population Increase in population Percentage increase in population

1940 8000 ---

1950 12000 4000 4000 x 100 = 50%

8000

1960 17000 5000 5000 x 100 = 41.7%

12000

1970 22500 5500 5500 x 100 = 32.4%

17000

TOTAL 14500 124.1%

AVERAGE 4833 41.37%

The population at the end of various decades shall be as follows:

# YEAR EXPECTED POPULATION

1980 22500 + 41.37 / 100 x 22500 = 31808

1990 31800 + 41.37 / 100 x 31800 = 49935

2000 49935 + 41.37 / 100 x 49395 = 68524

# INCREMENTAL INCREASE METHOD

This method is improvement over the above two methods. The average increase in the population is determined by the arithmetical method and to this is added the average of the net incremental increase once for each future decade.

# Solution:

Year Population Increase in populationIncrementalincrease

1940 8000 --- ---

1950 12000 4000 ---

1960 17000 5000 + 1000

1970 22500 5500 + 1500

TOTAL 14500 + 2500

AVERAGE 4833 1,250

The population at the end of the various decades shall be as follows:

# YEAR EXPECTED POPULATION

1980 22500 + (4833 + 1250) x 1 = 28583

1990 22500 + (4833 + 1250) x 2 = 34666

2000 22500 + (4833 + 1250) x 3 = 40749

# SOURCES OF WATER SUPPLY GENERAL INTRODUCTION

Water is the most abundant compound in nature. It covers 75% of the earth surface.About 97.3% of water is contained in the great oceans that are saline and 2.14% is held in icecaps glaciers in the poles, which are also not useful. Barely the remaining 0.56%  found on earth is in useful form for general livelihood. Total quantity of water

available on the planet “EARTH” in various states and religions are given in the table

# LOCATION VOLUME (m3) % OF TOTAL

Land Areas Fresh water lakes Saline lakes

Inland seas, Rivers Soil moisture Ground water Icecaps and glaciers TOTAL

Atmosphere

Water vapour (clouds) Oceans

Water in the oceans TOTAL ON PLANET 152 x 1012

104 x 1012

1.25 x 1012

67 x 1012

8350 x 1012

52 x 1012

37,800 x 1012

13 x 1012

13,20,000 x 1012

13,60,000 x 1012

0.009

0.008

0.001

0.005

0.005

0.610

2.80

0.001

97.3

100

# WATER ON THE PLANET HYDROLOGICAL CONCEPTS

Hydrology is the science, which deals with the increment of the water on the ground, under the ground, evaporation from the land and water surface and transportation from the vegetation and going back into atmosphere where it precipitates.

# DEFINITION

The water which goes in atmosphere by evaporation and transpiration again

comes back in the form of precipitation under favourable climatic conditions is known as hydrological cycle of water.

Due to sun’s heat water from the earth’s surfaces, lakes, rivers, seas etc evaporates and rises upwards. At high altitude due to reduction in the atmosphere pressure these water vapours expand by absorbing energy from the surrounding air, which cools down. When it falls below the dew point it cannot retain the excessive moisture, which starts falling in the form of rain, hails, dew, sleet, frost or precipitation. Various factors such as Temperature, atmospheric pressure, velocity of wind, height of mountains in the region, presence of forests, position of land and water areas etc and their complex relation are responsible for the precipitation. This precipitation and evaporation processes continue Forever and balance is maintained between the two by nature.

# PRECIPITATION

The evaporated water from the surfaces of streams, rivers, sea, ponds, wet

surfaces, trees and plants etc again returned to the earth surface by the condensation in the form of rain, hails, dew, sleet etc is known as precipitation. The major part of the precipitation occurs in the form of rain and other forms quantities are very small. The water of precipitation further goes off in the following ways.

- RUN-OFF: After precipitation a portion of its water flows over the ground in the form of rivers and streams and some water flows towards lakes and ponds and collected there.

- INFILTRATION: A portion of precipitation, percolates in the ground and it is stored in the form of sub-soil or ground water.

- EVAPORATION: some portion of the precipitation is also evaporated from the lakes, rivers, reservoirs and wet surfaces in the form of vapour due to sun’s heat is known as evaporation

- EVAPO-TRANSPIRATION: The roots of the trees sucks water from the ground and some portion of it evaporates in the atmosphere through leaves in the form of transpiration.

# SURFACE SOURCES

All the sources of water can be broadly divided into

- Surfaces sources and

- Sub surface sources

The surface sources further divided into

  - Streams

  - Rivers

  - Ponds

  - Lakes

  - Impounding reservoirs etc.

# NATURAL PONDS AND LAKES

In mountains at some places natural basin’s are formed with impervious bed by

springs and streams are known as “lakes”. The quality of water in the natural ponds and lakes depends upon the basin’s capacity, catchment area, annual rainfall, porosity of ground etc. But lakes and ponds situated at higher altitudes contain almost pure water which can be used without any treatment. But ponds formed due to construction of houses, road, railways conatins large amount of impurities and therefore cannot be used for water supply purposes.

# STREAMS AND RIVERS

Rivers and streams are the main source of surface source of water. In summer the quality of river water is better than mansoon.because in rainly season the run-off water also carries with clay, sand, silt etc which make the water turbid. So river and stream water require special treatments. Some rivers are snowfed and perennial and have water throughout the year and therefore they donot require any arrangements to hold the water. But some rivers dry up wholly or partially in summer. So they require special arrangements to meet the water demand during hot weather. Mostly all the cities are

situated near the rivers discharge their used water of sewege in the rivers, therefore much care should be taken while drawing water from the river.

# IMPOUNDING RESERVOIRS

In some rivers the flow becomes very small and cannot meet the requirements of hotweather. In such cases, the water can be stored by constructing a bund, a weir or a dam across the river at such places where minimum area of land is submerged in the water and max. quantity of water to be stored. In lakes and reservoirs, suspended impurities settle down in the bottom, but in their beds algae, weeds, vegetable and organic growth takes place which produce bad smell, taste and colour in water. Therefore this water should be used after purification. When water is stored for long time in reservoirs it should be aerated and chlorinated to kill the microscopic organisms which are born in water.

# SUBSURFACE SOURCES

These are further divided into

- Infiltration galleries

- Infiltration wells

- Springs etc

# INFILTRATION GALLERIES

A horizontal nearly horizontal tunnel which is constructed through water bearing strata for tapping underground water near rivers, lakes or streams are called “Infiltration galleries”. The yield from the galleries may be as much as 1.5 x 104 lit/day/metre length of infiltration gallery. For maximum yield the galleries may be placed at full depth of the acquifer. Infiltration galleries may be constructed with masonary or concrete with

weep holes of 5cm x 10cm.

# INFILTRATION WELLS

In order to obtain large quantity of water, the infiltration wells are sunk in series in the blanks of river. The wells are closed at top and open at bottom. They are constructed by brick masonary with open joints

# Infiltration Well

For the purpose of inspection of well, the manholes are provided in the top cover.

The water filtrates through the bottom of such wells and as it has to pass through sand bed, it gets purified to some extent. The infiltration well inturn are connected by porous pipes to collecting sump called jackwell and there water is pumped to purification plant for treatment.

# SPRINGS:

Sometimes ground water reappears at the ground surface in the form of springs.

Springs generally supply small springs. Springs generally supply small quantity of water and hence suitable for the hill towns. Some springs discharge hot water due to presence of sulphur and useful only for the curve of certain skin disease patients.

# Types of springs:

- Gravity Springs: When the surface of the earth drops sharply the water bearing stratum is exposed to atmosphere and gravity springs are formed

- Surface Spring: This is formed when an impervious stratum which is supporting the ground water reservoir becomes out crops

- Artesian Spring: When the ground water rises through a fissure in the upper impervious stratum When the water-bearing stratum has too much hydraulic gradient and is closed

between two imperious stratum, the formation of Artesian spring from deep seated spring

# WELLS:

A well is defined as an artificial hole or pit made in the ground for the purpose of tapping water. In India 75 to 85% of Indian population has to depend on wells for its water supply.

The three factors which form the basis of theory of wells are

- Geological conditions of the earth’s surface

- Porosity of various layers

- Quantity of water, which is absorbed and stored in different layers. The following are different types of wells

- Shallow wells

- Deep wells

- Tube wells

- Artesian wells

# Shallow Wells :

Shallow wells are constructed in the uppermost layer of the earth’s

surface. The diameter of well varies from 2 to 6 m and a maximum depth of 7m.

Shallow wells may be lined or unlined from inside. . These wells are also called draw wells or gravity wells or open wells or drag wells or percolation wells.

Quantity of water available from shallow wells is limited as their source of supply

is uppermost layer of earth only and sometimes may even dry up in summer. Hence they are not suitable for public water supply schemes. The quantity of water obtained from shallow wells is better than the river water but requires purification. The shallow wells should be constructed away from septic tanks, soak pits etc because of the contamination of effluent The shallow wells are used as the source of water supply for small villages, undeveloped municipal towns, isolated buildings etc because of limited supply and bad quality of water.

# Deep Wells :

The Deep wells obtain their quota of water from an aquifer below the impervious

layer as shown in fig No. The theory of deep well is based on the travel of water from the outcrop to the site of deep well. The outcrop is the place where aquifer is exposed to the atmosphere. The rain water entered at outcrop and gets thoroughly purified when it reaches to the site of deep well. But it dissolves certain salts and therefore become hard. In such cases, some treatment would be necessary to remove the hardness of water.

The depth of deep well should be decided in such a way that the location of out crop is not very near to the site of well. The water available at a pressure greater atmospheric pressure, therefore deep wells are also reffered to as a pressure wells. INTAKES FOR COLLECTING SURFACE WATER:

The main function of the intakes works is to collect water from the surface source and then discharge water so collected, by means of pumps or directly to the treatment water.

Intakes are structures which essentially consists of opening, grating or strainer

through which the raw water from river, canal or reservoir enters and carried to the sump well by means of conducts water from the sump well is pumped through the rising mains to the treatment plant.

The following points should be kept in mind while selecting a site for intake works.

- Where the best quality of water available so that water is purified economically in less time.

- At site there should not be heavy current of water, which may damage the intake structure.

- The intake can draw sufficient quantity of water even in the worest condition, when the discharge of the source is minimum.

- The site of the work should be easily approchable without any obstruction

- The site should not be located in navigation channels

- As per as possible the intake should be near the treatment plant so that conveyance cost is reduced from source to the water works

- As per as possible the intake should not be located in the vicinity of the point of sewage disposal for avoiding the pollution of water.

- At the site sufficient quantity should be available for the future expansion of the water-works.

# Types of Intake structures:

Depending upon the source of water the intake works are classified as following

- Lake Intake

- Reservoir Intake

- River Intake

- Canal Intake

# LAKE INTAKE:

For obtaining water from lakes mostly submersible intakes are used. These

intakes are constructed in the bed of the lake below the water level; so as to draw water in dry season also. These intakes have so many advantages such as no obstruction to the navigation, no danger from the floating bodies and no trouble due to ice. As these intakes draw small quantity of water, these are not used in big water supply schemes or on rivers

or reservoirs. The main reason being that they are not easily approachable for maintenance.

# RIVER INTAKE:

Water from the rivers is always drawn from the upstream side, because it is free

from the contamination caused by the disposal of sewage in it. It is circular masonary tower of 4 to 7 m in diameter constructed along the bank of the river at such place from where required quantity of water can be obtained even in the dry period. The water enters in the lower portion of the intake known as sump well from penstocks.

# RESERVOIR INTAKE:

Fig 3.13 shows the details of reservoir intake. It consists of an intake well, which is placed near the dam and connected to the top of dam by foot bridge.

The intake pipes are located at different levels with common vertical pipe. The

valves of intake pipes are operated from the top and they are installed in a valve room. Each intake pipe is provided with bell mouth entry with perforations of fine screen on its surface. The outlet pipe is taken out through the body of dam. The outlet pipe should be suitably supported. The location of intake pipes at different levels ensures supply of water from a level lower than the surface level of water.

When the valve of an intake pipe is opened the water is drawn off from the

reservoir to the outlet pipe through the common vertical pipe. To reach upto the bottom of intake from the floor of valve room, the steps should be provided in Zigzag manner.

# CANAL INTAKE:

Fig 3.14 shows the details of canal intake. A intake chamber is constructed in the

canal section. This results in the reduction of water way which increases the velocity of flow. It therefore becomes necessary to provide pitching on the downstream and upstream portion of canal intake.

The entry of water in the intake chamber takes through coarse screen and the top

of outlet pipe is provided with fine screen. The inlet to outlet pipe is of bell-mouth shape with perforations of the fine screen on its surface. The outlet valve is operated from the top and it controls the entry of water into the outlet pipe from where it is taken to the treatment plant.

# SUMMARY

- Sources of water supply are classified as

- Surface sources

- Sub surface sources

- Surface sources include rainfall, lakes, ponds, rivers and reservoirs etc.

- Subsurface sources are wells, springs, infiltration galleries

- Surface water is withdrawn by constructing intake structure

- Intake structures are classified as

- Lake intake

- Reservoir intake

- River intake

- Canal intake

Water Supply Engineering

- 28 -

- Wells are classified as

- Shallow wells

- Deep wells

- Deep wells get their supply from more than one water bearing stratum

- Infiltration galleries are constructed below the river bed to draw water during non mansoon season.

# SHORT ANSWER QUESTIONS

- what is hydrological cycle?

- List the sources of water supply.

- Mention different types of intakes.

- What is shallow well?

- What is infiltration gallery?

- Define spring.

- What are the different types of springs?

- Name the types of wells.

- Define deep well.

- What is precipitation?

- What is the purpose of intake structure?

# ESSAY TYPE QUESTIONS

- Explain the sources of water.

- Explain the classification of wells.

- What are points should be kept in mind while selecting a site for intake structure?

- Explain any one of intake structure with neat sketch.

# QUALITY OF WATER GENERAL INTRODUCTION

Absolutely pure water is never found in nature and contains number of impurities

in varying amounts. The rainwater which is originally pure, also absorbs various gases, dust and other impurities while falling. This water when moves on the ground further carries salt, organic and inorganic impurities. So this water before supplying to the public should be treated and purified for the safety of public health, economy and protection of various industrial process, it is most essential for the water work engineer to thoroughly check analyse and do the treatment of the raw water obtained the sources, before its distribution. The water supplied to the public should be strictly according to the standards laid down from time to time.

# CHARACTERISTICS OF WATER

For the purpose of classification, the impurities present in water may be divided into the following three categories.

# PHYSICAL CHARACTERISTICS

The following are the physical characteristics

- Turbidity

- Colour and temperature

- Taste and odour

# TURBIDITY

Turbidity is caused due to presence of suspended and colloidal matter in the

water. The character and amount of turbidity depends upon the type of soil over which the water has moved ground waters are less turbed than the surface water.

Turbidity is a measure of resistance of water to the passage of light through it. Turbidity is expressed as NTU (Nephelometric Turbidity Units) or PPM (parts per million) or Milligrams per litre (mg/l). Turbidity is measured by

1) Turbidity rod or Tape 2) Jacksons Turbidimeter 3) Bali’s Turbidimeter The Sample to be tested is poured into a test tube and placed in the meter and units of turbidity is read directly on the scale by a needle or by digital display.

Drinking water should not have turbidity more than 10 N.T.U. This test is useful

in determining the detension time in settling for raw water and to dosage of coagulants required to remove turbidity.

# COLOUR AND TEMPERATURE

Colour in water is usually due to organic matter in colloidal condition but some

times it is also due to mineral and dissolved organic impurities. The colour produced by one milligram of platinum in a litre of water has been fixed as the unit of colour. The permissible colour for domestic water is 20ppm on platinum cobalt scale. The colour in water is not harmful but objectionable.

Temperature of water is measured by means of ordinary thermometers. The temperature of surface water is generally at atmospheric temperature, while that of ground water may be more or less than atmospheric temperature. The most desirable

temperature for public supply between 4.4°C to 10°C. The temperature above 35°C are unfit for public supply, because it is not palatable.

# TASTE AND ODOUR

Taste and odour in water may be due to presence of dead or live micro-organisms, dissolved gases such as hydrogen sulphide, methane, carbon dioxide or oxygen combined with organic matter, mineral substances such as sodium chloride, iron compounds and carbonates and sulphates of other substances. The tests of these are done by sense of smell and taste because these are present in such small proportions that it is difficult to detect them by chemical analysis. The water having bad smell and odour is objectionable and should not be supplied to the public.

The intensities of the odours are measured in terms of threshold number. This number is numerically equal to the amount of sample of water in C.C’s required to be added to one litre of fresh odourless water.

# CHEMICAL CHARACTERISTICS

In the chemical analysis of water, these tests are done that will reveal the sanitary quality of the water. Chemical tests involve the determination of total solids, PH value, Hardness of water, Chloride content etc.

# TOTAL SOLIDS AND SUSPENDED SOLIDS

Total solids includes the solids in suspension colloidal and in dissolved form. The quantity of suspended solids is determined by filtering the sample of water through fine filter, drying and weighing. The quantity of dissolved and colloidal solids is determined by evaporating the filtered water obtained from the suspended solid test and weighing the residue. The total solids in a water sample can be directly determined by evaporating the filtered water obtained from the suspended solid test and weighing the residue. The total solids in a water sample can be directly determined by evaporating the water and weighing the residue of the residue of total solids is fused in a muffle furnace the organic

solids will decompose where as only inorganic solids will remain. By weighing we can determine the inorganic solids and deducting it from the total solids, we can calculate organic solids.

# PH VALUE OF WATER

PH value denotes the concentration of hydrogen ions in the water and it is a measure of acidity or alkanity of a substance.

PH = - log 10[H+] or 1 / log 10[H+]

Acidity ← Neutral→ Alkalinity

Depending upon the nature of dissolved salts and minerals, the PH value ranges

from 0 to 14. For pure water, PH value is 7 and 0 to 7 acidic and 7 to 14 alkaline range. For public water supply PH value may be 6.5 to 8.5. The lower value may cause tubercolation and corrosion, where as high value may produce incrustation, sediment deposits and other bad effects.

PH value of water is generally determined by PH papers or by using PH meter. PH can read directly on scale or by digital display using PH meter.

# HARDNESS OF WATER

It is a property of water, which prevents the lathering of the soap. Hardness is of two types.

- Temporary hardness: It is caused due to the presence of carbonates and sulphates of calcium and magnesium. It is removed by boiling.

- Permanent hardness: It is caused due to the presence of chlorides and nitrates of calcium and magnesium. It is removed by zeolite method.

Hardness is usually expressed in gm/litre or p.p.m. of calcium carbonate in water. Hardness of water is determined by EDTA method. For potable water hardness ranges from 5 to 8 degrees.

# HARDNESS REMOVABLE

Generally a hardness of 100 to 150 mg/litre is desirable. Excess of hardness leads to the following effects.

- Large soap consumption in washing and bathing

- Fabrics when washed become rough and strained with precipitates.

- Hard water is not fit for industrial use like textiles, paper making, dye and ice cream manufactures.

- The precipitates clog the pores on the skin and makes the skin rough

- Precipitates can choke pipe lines and values

- It forms scales in the boilers tubes and reduces their efficiency and cause in erustations

- Very hard water is not palatable

When softening is practices when hardness exceed 300mg/lit. Water hardness more than 600 mg/lit have to rejected for drinking purpose.

# METHODS OF REMOVAL OF HARDNESS

- Boiling

- Freezing

- Lime addition

- Lime soda process

- Excess Lime treatment

- Caustic soda process

- Zeolete process

- Dimineralisation or exchange process.

Methods 1,2 and 3 are suitable for removal of temporary hardness and 4 to 8 for both temperory and permanent hardness. The temporary hardness is removed as follows.

# Boiling

heat

Ca(HCO3)2	> CaCO3↓ + CO2↑ +H2O

heat

Mg(HCO3)2	> MgCO3↓ + CO2↑ +H2O

# Addition of lime

Ca (HCO3)2 + Ca(OH)2	> 2CaCO3↓ + 2H2O

Mg(HCO3)2 + Ca(OH)2	> CaCO3 + MgCO3 + 2H2O

# Removal of permanent Hardness:

1. Lime soda process : In this method, the lime and is sodium carbonate or soda as have used to remove permanent hardness from water. The chemical reactions involved in this process are as follows.

CO2 + Ca(OH)2	> CaCO3 + H2O (removal of CO2)

Ca(HCO3) + Ca(OH)2	> 2CaCO3 + 2H3O (removal of temporary hardness)

Mg(HCO3) + Ca(OH)2	> CaCO3 + Mg(CO3) + 2H2O

MgSO4 + Ca(OH)2	> Mg(OH)2 + CaSO4 { conversion of MgSO4 to CaSO4}

CaSO4 + Na2CO3	> CaCO3 + Na2SO4 {removal of sulphates}

CaCl2 + Ca(OH)2	> Ca(OH)2 + CaCl2

MgCl2 + Ca(OH)2	> Mg(OH)2 + CaCl2 { removal of chlorides}

CaCl2 + Na2CO3	> CaCO3 + 2NaCl

MgCl2 + Na2CO3	> Mg CO3 + 2NaCl {removal of chlorides}

# Advantages of lime soda process

- The PH value of water treated by this process bring down to 9 and which results in decrease in corrosion of the distribution system.

- Less quantity of coagulant will be required, if this process is adopted

- Removal of iron and manganese to some extent

- Reduction of total mineral content of water

- Hardness of water is reduced to 40mg/lit (of CaCO3) and magnesium upto 10mg/lit

- The process is economical

- This process is most suitable for tubed and acidic waters where it will not possible to adopt zeolite process.

# Disadvantages

- Large quantity of sludge formed during this process to be disposed off by some suitable method

- This process requires skilled supervision for its successful working

- If recarbonation is omitted, a thick layer of calcium carbonate will be deposited in the filtering media, distribution pipes etc.

# Zeolite process

This is also known as the base-exchange or Ion exchange process. The hardness may be completely removed by this process.

# Principle

Zeolites are compounds (silicates of aluminium and sodium) which replace

sodium Ions with calcium and magnesium Ions when hardwater is passes through a bed of zeolites. The zeolite can be regenerated by passing a concentrated solution of sodium chloride through the bed. The chemical reactions involved are

2SiO2 Al2O3 Na2O + Ca(HCO3)2	> 2SiO2 Al2O3 CaO + 2NaHCO3

(Zeolite)

2SiO2 Al2O3 Na2O + CaSO4	> 2SiO2 Al2O3 CaO + Na2SO4

2SiO2 Al2O3 Na2O + CaC12	> 2SiO2 Al2O3 CaO + 2NaCl

# Regeneration

2SiO2 Al2O3 Na2O + 2NaCl	> 2SiO2 Al2O3 Na2O + CaCl2

2SiO2 Al2O3 MgO + 2NaCl	> 2SiO2 Al2O3 Na2O + MgCl2

# Advantages

- In this process, the sludge is not formed hence problem of sludge disposal does not arise

- It can be operated easily and no skilled supervision required

- The hardness of water reduces to zero and hence used for boiler and texile industries

- The process is economical where salt is cheaply available

- The load on Zeolite can be reduced by combining it with lime or aeration process

# Disadvantages

- The Zeolite process cannot be used for turbed or acidic water

- The Zeolite process is unsuitable for water containing Iron and Manganese

- The Zeolite should be operated carefully to avoid injury or damage to the equipment

# Demineralisation

Both cations and anions are removed by resins similar to zeolites in two columns

by iron exchange method. Resins may be regenerated with sulpuric acid and sodium carbonate. This process is used in industries to get distilled water or quality water motion of water through the atmosphere, earth, plants, trees, rivers and oceans in a cyclic motion through liquid, solid and gaseous phases is called HYDROLOGICAL CYCLE. CHLORIDE CONTENT

The natural waters near the mines and sea dissolve sodium chloride and also presence of chlorides may be due to mixing of saline water and sewage in the water. Excess of chlorides is dangerous and unfit for use. The chlorides can be reduced by diluting the water. Chlorides above 250p.p.m. are not permissible in water.

# NITROGEN CONTENT

The presence of nitrogen in the water indicates the presence of organic matters in the water. The nitrogen may be present in the water may be in one or more of the following forms.

1. Nitrates 2. Nitrates 3. Free ammonia 4. Albuminoid nitrogen.

Excess presence of nitrogen will cause “MATHEMOGLOBINEMIA” disease to the

children.

# METALS AND OTHER CHEMICAL SUBSTANCES

Water contains various minerals or metal substances such as iron, manganese, copper, lead, barium, cadmium, selenium, fluoride, arsenic etc.

The concentration of iron and manganese should not allow more than 0.3 ppm .

Excess will cause discolouration of clothes during washing and incrustation in water mains due to deposition of ferric hydroxide and manganese oxide. Lead and berium are very toxic, low p.p.m of these are allowed. Arsenic, Selenium are poisonous and may cause totally, therefore they must be removed totally. Human beings are effected by presence of high quality of copper in the water. Fewer cavities in the teeth will be formed due to excessive presence of fluoride in water more than 1 p.p.m. A laxative effect is caused in the human body due to excessive presence of sulphate in the water.

# DISSOLVED GASES

oxygen and carbondi-oxide are the gases mostly found in the natural water. The

surface water contain large amount of dissolved oxygen because they absorb it from the atmosphere. Algae and other tiny plant life of water also give oxygen to the water. The presence of oxygen in the water in dissolved form keep it fresh and sparkling. But more quantity of oxygen causes corrosion to the pipes material.

Water absorbs carbon-dioxide from the atmosphere. If water comes across

calcium and magnesium salts, carbon-dioxide reacts with the salts and converts them into bicarbonates, causes hardness in the water. The presence of carbon-dioxide is easily determined by adding lime solution to water gives milky white colour.

# BIO-CHEMICAL OXYGEN DEMAND

If the water is contaminated with sewage, the demand of oxygen by organic

matter in sewage is known as biochemical oxygen demand. The aerobic action continues till the oxygen is present in sewege. As the oxygen exhausts the anerobic action begins due to which foul smell starts coming. Therefore indirectly the decomposable matters require oxygen, which is used by the organisms.

The aerobic decomposition of organic matters is done in two stages. The

carbonaceous matters are first oxidized and the oxidation of nitrogeneous matters takes place in the latter stage.

# BACTERIAL AND MICROSCOPICAL CHARACTERISTICS

The examination of water for the presence of bacteria is important for the water supply engineer from the viewpoint of public health. The bacteria may be harmless to mankind or harmful to mankind. The former category is known as non-pathogenic bacteria and the later category is known as pathogenic bacteria. Many of the bacteria found in water are derived from air, soil and vegetation. Some of these are able to

multiply and continue their existence while the remaining die out in due course of time. The selective medium that promote the growth of particular bacteria and inbuilt the growth of other organisms is used in the lab to detect the presence of the required bacteria, usually coliform bacteria. For bacteriological analysis the following tests are done.

# PLANT COUNT TEST

In this method total number of bacteria presents in a millitre of water is counted. 1

ml of sample water is diluted in 99ml of sterilized water and 1ml of dilute water is mixed with  10ml  of  agar  of  gelatine.  This  mixture  is  then  kept  in  incubator  at  37°C  for  24 hours

or 20°C for 48 hours. After the sample will be taken out from the incubator and colonies of bacteria are counted by means of microscope.

Drinking water should not have more than 10 coliforms/100ml.

# M.P.N. TEST (MOST PROBABLE NUMBER)

The detection of bacteria by mixing different dilutions of a sample of water with fructose broth and keeping it in the incubator at 37°C for 48hours. The presence of acid or carbon-dioxide gas in the test tube will indicate the presence of B-coli. After this the standard statistical tables (Maccardy’s) are reffered and the “MOST PROBABLE NUMBER” (MPN) of B-coli per 100ml of water are determined.

For drinking water, the M.P.N. should not be more than 2.

# WATER BORNE DISEASES

World health organization has observes that 80% of communicable diseases that are transmitted through water. The diseases like cholera, gastroenteritis, typhoid,

amoebia, diarrhoea, polio, hepatitis (Jaundice), Leptospirosis, Dracontiasis are caused by bacteria.

Excess of fluorides present in water [ above 1.5 mg/litre] cause diseases like

dental flurosis, sketetal flurosis. This is a permanent irresible disease that weakens the bone structure. The patient becomes immobile and bedridden.

Excess of nitrates in water causes Mathaemoglobinaemia or blue baby symptoms

in infants. It effects the hemoglobin in the blood and reduces its capacity to transport oxygen to the cells. Nitrates in water are caused by industrial effluents, agricultural runoff. Toxic ions of chromium, lead, arsenic and pesticides in water cause diseases affecting the kidney, liver and high blood pressure, paralysis, cancer etc. These toxic substances are due to industrial effluents reaching the surface and ground water sources.

# DRINKING WATER STANDARDS

S.No. CHARACTERICTICS NORMALLY ACCEPTABLE

VALUE MAX. PERMISIBLE LIMIT

- Temperature 10°C – 15°C -

- Turbidity (N.T.U) 2.5 10

- Colour (platinum cobalt scale) 5.0 25

- Taste and odour Unobjectionable 5. PH 7.0-8.5 6.5-9.2

- Total dissolved solids(mg/litre) 500 1500

- Total hardness mg/l (as caco3) 200 600

- Chlorides (as Cl) mg/l 200 1000

- Sulphates (as So4) mg/l 200 400

- Nitrates ( as No3) mg/l 45 45

- Fluorides (as F) mg/l 1.0 1.5

- Calcium (as Ca) mg/l 75 200

- Magnesium (as mg) mg/l 30-120 150

- Iron (as Fe) mg/l 0.1 1.0

- Manganese (As Mn) mg/l 0.05 0.5

- Phenolic compounds (as phenol) mg/l

0.001 0.002

- Arsenic (as mg) mg/l 0.05 0.05

- Chromium (as cr+6) mg/l 0.05 0.05

- Cynamides (as CN) mg/l 0.05 0.05

- Coliform count per 100ml of water sample

# SHORT ANSWER QUESTIONS

- Name the different characteristics of water.

- What are the physical characteristics of water?

- What is the unit for the measurement of turbidity?

- Define hardness of water.

- What is temporary hardness of water?

- What is permanent hardness of water?

- Name any four chemical characteristics of water.

- Define PH.

- What are effects of excess nitrogen content?

- What are the metals present in water?

- Name the dissolved gases in the water.

- Define B.O.D.

- Name the tests to be conducted for bacteriological tests.

- Name any four water borne diseases.

- What is the effect of excess floride in water?

- State any four parameters of drinking water quality standard.

- What is the range of PH value of domestic water supply?

# ESSAY TYPE QUESTIONS

- Explain different characteristics of water.

- What is the turbidity? How it is measured?

- Define PH of water. Explain the significance of PH.

- What is significance of biological tests? Explain M.P.N. test?

- List out the different water borne diseases explain the effects of excess fluoride & nitrogen contents.

- How total solids are determined?

- List out the drinking water standards.